4. OSI Transport Layer

introduction

Data  networks  and  the  Internet  support  the  human  network  by  supplying  seamless, reliable communication between people - both locally and around the globe. On a single device, people  can  use  multiple  services  such  as  e-mail,  the  web,  and  instant  messaging  to  send messages or retrieve information. Applications such as e-mail clients, web browsers, and instant messaging  clients  allow  people  to  use  computers  and  networks  to  send  messages  and  find information.

Data  from  each  of  these  applications  is  packaged,  transported,  and  delivered  to  the appropriate server daemon or application on the destination device. The processes described in

the OSI Transport layer accept data from the Application layer and prepare it for addressing at

the  Network  layer.  The  Transport  layer  is  responsible  for  the  overall  end-to-end  transfer  of application data.

In this chapter, we examine the role of the Transport layer in encapsulating application data for use by the Network layer. The Transport layer also encompasses these functions:

• Enables multiple applications to communicate over the network at the same time

on a single device.

•  Ensures  that,  if  required,  all  the  data  is  received  reliably  and  in  order  by  the correct application.

• Employs error handling mechanisms.

Learning Objectives

Upon completion of this chapter, you will be able to:

	Explain the need for the Transport layer.

	Identify  the  role  of  the  Transport  layer  as  it  provides  the  end-to-end  transfer  of  data between applications.

	Describe the role of two TCP/IP Transport layer protocols: TCP and UDP.

	Explain  the  key  functions  of the  Transport  layer,  including  reliability,  port  addressing, and segmentation.

	Explain how TCP and UDP each handle key functions.

	Identify when it is appropriate to use TCP or UDP and provide examples of applications that use each protocol.

4.1 Roles  of the Transport Layer

4.1.1 Purpose of the Transport Layer

The  Transport  layer provides  for the  segmentation of data and the  control necessary to reassemble these pieces into the various communication streams. Its primary responsibilities to accomplish this are:

- Tracking	the	individual   communication	between   applications	on   the	source	and destination hosts.

- Segmenting data and managing each piece.

- Reassembling the segments into streams of application data.

- Identifying the different applications.

- Data Requirements Vary.

- Separating Multiple Communications.

Tracking Individual Conversation

Any  host  may  have  multiple  applications  that  are  communicating  across  the  network. Each  of  these  applications  will  be  communicating  with  one  or  more  applications  on  remote hosts.  It  is  the  responsibility  of  the  Transport  layer  to  maintain  the  multiple  communication streams between these applications.

Segmenting Data

As  each  application  creates  a  stream  data  to  be  sent  to  a  remote  application,  this  data must  be  prepared  to  be  sent  across  the  media  in  manageable  pieces.  The  Transport  layer protocols describe services that segment this data from the Application layer. This includes the encapsulation required on each piece of data. Each piece of application data requires headers to

be added at the Transport layer to indicate to which communication it is associated.

Reassembling Segments

At the receiving host, each piece of data may be directed to the appropriate application. Additionally,  these  individual pieces  of data  must  also  be  reconstructed  into  a  complete  data stream that is useful to the Application layer. The protocols at the Transport layer describe the how the Transport layer header information is used to reassemble the data pieces into streams to

be passed to the Application layer.

Identifying the Applications

In order to pass data streams to the proper applications, the Transport layer must identify

the  target  application.  To  accomplish  this,  the  Transport  layer  assigns  an  application  an identifier. The TCP/IP protocols call this identifier a port number. Each software process that needs to access the network is assigned a port number unique in that host. This port number is used  in  the  transport  layer  header  to  indicate  to  which  application  that  piece  of  data  is associated.

The Transport layer is the link between the Application layer and the lower layer that are

responsible for network transmission. This layer accepts data from different conversations and passes it down to the lower layers as manageable pieces that can be eventually multiplexed over

the media.

Applications  do  not  need  to  know  the  operational  details  of  the  network  in  use.  The

applications  generate  data  that  is  sent  from  one  application  to  another,  without  regard  to  the destination host type, the type of media over which the data must travel, the path taken by the data, the congestion on a link, or the size of the network.

Additionally, the lower layers are not aware that there are multiple applications sending data  on  the  network.  Their  responsibility  is  to  deliver  data  to  the  appropriate  device.  The Transport layer then sorts these pieces before delivering them to the appropriate application.

Data Requirements Vary

Because different applications have different requirements, there are  multiple Transport layer  protocols.  For  some  applications,  segments  must  arrive  in  a  very  specific  sequence  in order to be processed successfully. In some cases, all of the data must be received for any of it

to be of use. In other cases, an application can tolerate some loss of data during transmission

over the network.

In today's converged  networks, applications with  very different transport  needs  may be communicating  on  the  same  network.  The  different  Transport  layer  protocols  have  different rules allowing devices to handle these diverse data requirements.

Some protocols provide just the basic functions for efficiently delivering the data pieces

between the appropriate applications. These types of protocols are useful for applications whose data is sensitive to delays.

Other Transport layer protocols describe processes that provide additional features, such

as ensuring reliable delivery between the applications. While these additional functions provide more robust communication at the  Transport  layer between applications, they have  additional overhead and make larger demands on the network.

Separating Multiple Communications

Consider a computer connected to a network that is simultaneously receiving and sending

e-mail  and  instant  messages,  viewing  websites,  and  conducting  a  VoIP  phone  call.  Each  of these applications is sending and receiving data over the network at the same time. However, data from the phone call is not directed to the web browser, and text from an instant message does not appear in an e-mail.

Further, users require that an e-mail or web page be completely received and presented

for the  information to be considered  useful. Slight delays are considered acceptable to ensure that the complete information is received and presented.

In  contrast,  occasionally  missing  small  parts  of  a  telephone  conversation  might  be considered  acceptable.  One  can  either  infer  the   missing  audio  from  the  context  of  the conversation or ask the other person to repeat what they said. This is considered preferable to

the delays that would result from asking the network to manage and resend missing segments.

In this example, the user - not the network - manages the resending or replacement of missing

information.

As explained in a previous chapter, sending some types of data - a video for example - across a network as one complete communication stream could prevent other communications from occurring at the same time. It also makes error recovery and retransmission of damaged data difficult.

Dividing data into small parts, and sending these parts from the source to the destination, enables many different communications to be interleaved (multiplexed) on the same network.

Segmentation  of  the  data,  in  accordance  with  Transport  layer  protocols,  provides  the means  to  both  send  and  receive  data  when  running  multiple  applications  concurrently  on  a computer. Without segmentation, only one application, the streaming video for example, would

be able to receive data. You could not receive e-mails, chat on instant messenger, or view web pages while also viewing the video.

At the Transport layer, each particular set of pieces flowing between a source application

and a destination application is known as a conversation.

To  identify  each  segment  of  data,  the  Transport  layer  adds  to  the  piece  a  header containing binary data. This  header contains  fields of bits. It  is the  values in these  fields that enable different Transport layer protocols to perform different functions.

4.1 Roles  of the Transport Layer

4.1.2 Controlling the Conversations

The primary functions specified by all Transport layer protocols include:

Segmentation and Reassembly - Most networks have a limitation on the amount of data that can be included in a single PDU. The Transport layer divides application data into blocks

of data that are an appropriate size. At the destination, the Transport layer reassembles the data before sending it to the destination application or service.

Conversation Multiplexing  -  There  may  be  many applications  or  services  running  on each host in the network. Each of these applications or services is assigned an address known as

a port so that the  Transport  layer can determine with which application or service the data  is

identified.

In addition to using the information contained in the headers, for the basic functions of data segmentation and reassembly, some protocols at the Transport layer provide:

• Connection-oriented conversations

• Reliable delivery

• Ordered data reconstruction

• Flow control

Establishing a Session

The  Transport  layer  can  provide  this  connection  orientation  by  creating  a  sessions between the applications. These connections prepare the applications to communicate with each other  before  any  data  is  transmitted.  Within  these  sessions,  the  data  for  a  communication between the two applications can be closely managed.

Reliable Delivery

For  many  reasons,  it  is  possible  for  a  piece  of  data  to  become  corrupted,  or  lost

completely, as it is transmitted over the network. The Transport layer can ensure that all pieces reach their destination by having the source device to retransmit any data that is lost.

Same Order Delivery

Because networks may provide multiple routes that can have different transmission times, data can arrive in the wrong order. By numbering and sequencing the segments, the Transport layer can ensure that these segments are reassembled into the proper order.

Flow Control

Network  hosts  have  limited  resources,  such as  memory or bandwidth.  When Transport layer is aware that these resources are overtaxed, some protocols can request that the sending application reduce the rate of data  flow. This  is done at the  Transport  layer by regulating the amount of data the source transmits as a group. Flow control can prevent the loss of segments

on the network and avoid the need for retransmission.

As the protocols are discussed  in this chapter, these services will be explained  in more

detail.

4.1 Roles  of the Transport Layer

4.1.3 Supporting Reliable Communication

Recall that the primary function of the Transport layer is to manage the application data

for	the	conversations	between	hosts.	However,	different	applications	have	different requirements for their data, and therefore different Transport protocols have been developed to meet these requirements.

A Transport layer protocol can implement is a method to ensure reliable delivery of the data.  In  networking  terms,  reliability  means  ensuring  that  each  piece  of  data  that  the  source sends arrives at the destination. At the Transport layer the three basic operations of reliability are:

• tracking transmitted data

• acknowledging received data

• retransmitting any unacknowledged data

This requires the processes of Transport layer of the source to keep track of all the data pieces of each conversation and the retransmit any of data that did were not acknowledged by

the  destination.  The  Transport  layer  of  the  receiving  host  must  also  track  the  data  as  it  is received and acknowledge the receipt of the data.

These reliability processes place additional overhead on the network resources due to the acknowledgement,  tracking,  and  retransmission.  To  support  these  reliability operations,  more control data is exchanged between the sending and receiving hosts. This control information is contained in the Layer 4 header.

This creates a trade-off between the  value  of reliability and the burden it places on the

network.  Application  developers  must  choose  which  transport  protocol  type  is  appropriate based on the requirements of their applications. At the Transport layer, there are protocols that specify methods for either reliable, guaranteed delivery or best-effort delivery. In the context of networking,	best-effort	delivery	is	referred	to	as	unreliable,	because	there	is	no acknowledgement that the data is received at the destination.

Determining the Need for Reliability

Applications, such as databases, web pages, and e-mail, require that all of the sent data arrive at the destination in its original condition, in order for the data to be useful. Any missing data could cause a corrupt  communication that  is either incomplete or unreadable. Therefore, these applications are designed to use a Transport layer protocol that implements reliability. The additional network overhead is considered to be required for these applications.

Other applications are more tolerant of the loss of small amounts of data. For example, if

one  or  two  segments  of  a  video  stream  fail  to  arrive,  it  would  only  create  a  momentary disruption  in  the  stream.  This  may  appear  as  distortion  in  the  image  but  may  not  even  be noticeable to the user.

Imposing overhead to ensure reliability for this application could reduce the usefulness of

the  application.  The  image  in a  streaming  video  would  be  greatly degraded  if the  destination device  had  to  account  for  a  lost  data  and  delay  the  stream while  waiting  for  its  arrival.  It  is better to  render  the  best  image  possible  at  the  time  with  the  segments  that  arrive  and  forego reliability.  If  reliability  is  required  for  some  reason,  these  applications  can  provide  error checking and retransmission requests.

4.1 Roles  of the Transport Layer

4.1.4 TCP and UDP

The   two   most   common   Transport   layer   protocols   of   TCP/IP   protocol   suite   are Transmission  Control  Protocol  (TCP)  and  User  Datagram  Protocol  (UDP).  Both  protocols manage the communication of multiple applications. The differences between the two are the specific functions that each protocol implements.

User Datagram Protocol (UDP)

UDP is a simple, connectionless protocol, described in RFC 768. It has the advantage of providing  for  low  overhead  data  delivery.  The  pieces  of  communication  in  UDP  are  called datagrams. These datagrams are sent as "best effort" by this Transport layer protocol.

	Applications that use UDP include:

	Domain Name System (DNS).

	Video Streaming.

	Voice over IP (VoIP).

Transmission Control Protocol (TCP)

TCP  is  a  connection-oriented  protocol,  described  in  RFC  793.  TCP  incurs  additional overhead to gain functions. Additional functions specified by TCP are the same order delivery, reliable delivery, and flow control. Each TCP segment has 20 bytes of overhead in the header encapsulating  the  Application  layer  data,  whereas  each  UDP  segment  only  has  8  bytes  of overhead. See the figure for a comparison.

Applications that use TCP are:

• Web Browsers.

• E-mail.

• File Transfers.

4.1 Roles  of the Transport Layer

4.1.5 Port Addressing

Identifying the Conversations

Consider the earlier example of a computer simultaneously receiving and sending e-mail, instant messages, web pages, and a VoIP phone call.

The  TCP  and  UDP  based  services  keep  track  of  the  various  applications  that  are

communicating.  To  differentiate  the  segments  and  datagrams  for  each  application,  both  TCP

and  UDP  have  header  fields  that  can  uniquely  identify  these  applications.  These  unique identifiers are the port numbers,

In the  header of each segment  or datagram,  there  is  a source  and  destination port.  The source  port  number  is  the  number  for  this  communication  associated  with  the  originating application on the local host. The destination port number is the number for this communication associated with the destination application on the remote host.

Port  numbers  are  assigned  in  various  ways,  depending  on  whether  the  message  is  a

request or a response. While server processes have static port numbers assigned to them, clients dynamically choose a port number for each conversation.

When  a  client  application  sends  a  request  to  a  server  application,  the  destination  port

contained in the header is the port number that is assigned to the service daemon running on the remote  host.  The  client  software  must  know  what  port  number  is  associated  with  the  server process  on  the  remote  host.  This  destination  port  number  is  configured,  either  by  default  or manually. For example, when a web browser application makes a request to a web server, the browser uses TCP and port number 80 unless otherwise specified. This is because TCP port 80

is  the  default  port  assigned  to  web-serving  applications.  Many  common  applications  have default port assignments.

The  source  port  in  a  segment  or  datagram  header  of  a  client  request  is  randomly

generated. As long as it does not conflict with other ports in use on the system, the client can choose  any  port  number.  This  port  number  acts  like  a  return  address  for  the  requesting application.  The  Transport  layer keeps  track  of this  port  and  the  application that  initiated  the request so that when a response is returned, it can be forwarded to the correct application. The requesting  application  port  number  is  used  as  the  destination  port  number  in  the  response coming back from the server.

The combination of the  Transport  layer port  number and  the Network  layer IP address assigned to the host uniquely identifies a particular process running on a specific host device. This  combination  is  called  a  socket.  Occasionally,  you  may  find  the  terms  port  number  and socket  used  interchangeably.  In  the  context  of this  course,  the  term socket  refers  only  to  the unique combination of IP address and port number. A socket pair, consisting of the source and destination  IP  addresses  and  port  numbers,  is  also  unique  and  identifies  the  conversation between the two hosts.

For example, an HTTP web page request being sent to a web server (port 80) running on

a	host	with	a	Layer	3	IPv4	address	of	192.168.1.20	would	be	destined	to	socket

192.168.1.20:80.

If the web browser requesting the web page  is running on host 192.168.100.48 and the

Dynamic port number assigned to the web browser is 49152, the socket for the web page would

be 192.168.100.48:49152.

The  Internet  Assigned  Numbers  Authority  (IANA)  assigns  port  numbers.  IANA  is  a standards body that is responsible for assigning various addressing standards.

There are different types of port numbers:

Well Known Ports (Numbers 0 to 1023) - These numbers are reserved for services and applications.	They	are	commonly	used	for	applications	such	as	HTTP	(web	server) POP3/SMTP  (e-mail  server)  and  Telnet.  By  defining  these  well-known  ports  for  server applications, client applications can be programmed to request a connection to that specific port

and its associated service.

Registered Ports (Numbers 1024 to 49151) - These port  numbers are assigned to  user processes or applications. These processes are primarily individual applications that a user has chosen to install rather than common applications that would receive a Well Known Port. When

not used for a server resource, these ports may also be used dynamically selected by a client as

its source port.

Dynamic or Private Ports  (Numbers 49152 to 65535) - Also known as Ephemeral Ports,

these are usually assigned dynamically to client applications when initiating a connection. It is

not very common for a client to connect to a service using a Dynamic or Private Port (although some peer-to-peer file sharing programs do).

Using both TCP and UDP

Some applications may use both TCP and UDP. For example, the low overhead of UDP enables  DNS  to  serve  many  client  requests  very  quickly.  Sometimes,  however,  sending  the requested  information  may  require  the  reliability  of  TCP.  In  this  case,  the  well  known  port number of 53 is used by both protocols with this service.

Sometimes it is necessary to know which active TCP connections are open and running

on a  networked  host.  Netstat  is  an  important  network  utility that  can  be  used  to  verify those connections.  Netstat  lists  the  protocol  in  use,  the  local  address  and  port  number,  the  foreign address and port number, and the state of the connection.

Unexplained TCP connections can pose a major security threat. This is because they can indicate  that  something  or  someone  is  connected  to  the  local  host.  Additionally,  unnecessary TCP  connections  can  consume  valuable  system  resources  thus  slowing  down  the  host's performance.  Netstat  should  be  used  to  examine  the  open  connections  on  a  host  when performance appears to be compromised.

Many useful options are available for the netstat command.

4.1 Roles  of the Transport Layer

4.1.6 Segmentation and Reassembly – Divide and Conquer

A previous  chapter explained  how  PDUs  are  built  by passing data  from an application down through the various protocols to create a PDU that is then transmitted on the medium. At

the destination host, this process is reversed until the data can be passed up to the application.

Some  applications  transmit  large  amounts  of  data  -  in  some  cases,  many  gigabytes.  It would be impractical to send all of this data in one large piece. No other network traffic could

be transmitted while this data was being sent. A large piece of data could take minutes or even

hours to send. In addition, if there were any error, the entire data file would have to be lost or resent. Network devices would not have memory buffers large enough to store this much data while  it  is  transmitted  or received.  The  limit  varies  depending  on the  networking technology

and specific physical medium being in use.

Dividing  application data  into  pieces  both ensures  that  data  is  transmitted  within

the limits of the media and that data from different applications can be multiplexed on to the media.

TCP and UDP Handle Segmentation Differently

In TCP, each segment header contains a sequence number. This sequence number allows

the  Transport  layer  functions  on  the  destination  host  to  reassemble  segments  in  the  order  in which they were  transmitted.  This  ensures  that  the  destination application  has  the  data  in the exact form the sender intended.

Although services using UDP also track the conversations between applications, they are

not  concerned  with  the  order  in  which  the  information  was  transmitted,  or  in  maintaining  a connection.  There  is  no  sequence  number  in  the  UDP  header.  UDP  is  a  simpler  design  and generates less overhead than TCP, resulting in a faster transfer of data.

Information  may  arrive  in  a  different  order  than  it  was  transmitted  because  different packets  may  take  different  paths  through  the  network.  An  application  that  uses  UDP  must tolerate the fact that data may not arrive in the order in which it was sent.

The  key  distinction  between  TCP  and  UDP  is  reliability

The  reliability  of  TCP communication is performed using connection-oriented sessions. Before a host using TCP sends data  to  another  host,  the  Transport  layer  initiates  a  process  to  create  a  connection  with  the destination.  This  connection  enables  the  tracking  of  a  session,  or  communication  stream between  the  hosts.  This  process  ensures  that  each  host  is  aware  of  and  prepared  for  the communication. A complete TCP conversation requires the establishment of a session between the hosts in both directions.

After  a  session  has  been  established,  the  destination  sends  acknowledgements  to  the source for the segments that it receives. These acknowledgements form the basis of reliability within the TCP session. As the source receives an acknowledgement, it knows that the data has been successfully  delivered  and  can quit  tracking that  data.  If the  source  does  not  receive  an acknowledgement  within  a  predetermined  amount  of  time,  it  retransmits  that  data  to  the destination.

Part  of  the  additional  overhead  of  using  TCP  is  the  network  traffic  generated  by

acknowledgements and retransmissions. The establishment of the sessions creates overhead in

the  form  of  additional  segments  being  exchanged.  There  is  also  additional  overhead  on  the

individual  hosts  created  by  the  necessity  to  keep  track  of  which  segments  are  awaiting

acknowledgement and by the retransmission process.

This  reliability  is  achieved  by  having  fields  in  the  TCP  segment,  each  with  a  specific function, as shown in the figure. These fields will be discussed later in this section.

4.2 The TCP protocol- Communicating  with Reliability

4.2.2 TCP Server Processes

As  discussed  in  the  previous  chapter,  application  processes  run  on  servers.  These processes  wait  until  a  client  initiates  communication  with  a  request  for  information  or  other services.

Each application process running on the server is configured to use a port number, either

by  default  or  manually  by  a  system  administrator.  An  individual  server  cannot  have  two services  assigned  to  the  same  port  number  within the  same  Transport layer  services.  A host  running  a  web  server  application  and  a  file  transfer  application  cannot  have  both configured  to  use  the  same  port  (for  example,  TCP  port  8080).  When  an  active  server application is assigned to a specific port, that port is considered to be "open" on the server. This

means  that  the  Transport  layer  accepts  and  processes  segments  addressed  to  that  port.  Any

incoming client request addressed to the correct socket is accepted and the data is passed to the server application. There can be many simultaneous ports open on a server, one for each active server application. It is common for a server to provide more than one service, such as a web server and an FTP server, at the same time.

One way to improve security on a server is to restrict server access to only those ports associated with the services and applications that should be accessible to authorized requestors. The   figure   shows   the   typical   allocation   of   source   and   destination   ports   in   TCP

client/server operations.

4.2 The TCP protocol- Communicating  with Reliability

4.2.3 TCP Connection Establishment and Termination

When two hosts communicate using TCP, a connection is established before data can be exchanged. After the communication is completed, the sessions are closed and the connection is terminated. The connection and session mechanisms enable TCP's reliability function.

See the figure for the steps to establish and terminate a TCP connection.

The  host  tracks  each  data  segment  within  a  session  and  exchanges  information  about what data is received by each host using the information in the TCP header.

Each  connection  represents  two  one-way  communication  streams,  or  sessions.   To establish  the  connection,  the  hosts  perform  a  three-way  handshake.  Control  bits  in  the  TCP

header indicate the progress and status of the connection. The three-way handshake:

- Establishes that the destination device is present on the network.

- Verifies that the destination device has an active service and is accepting requests

on the destination port number that the initiating client intends to use for the session.

- Informs  the  destination  device  that  the  source  client  intends  to  establish  a communication session on that port number.

In TCP connections, the  host serving as a  client  initiates the session to  the server.  The three steps in TCP connection establishment are:

1. The initiating client sends a segment containing an initial sequence value, which serves

as a request to the server to begin a communications session.

2. The server responds with a segment containing an acknowledgement value equal to the received sequence  value  plus 1, plus  its own synchronizing sequence value. The  value  is one greater than the sequence number because there is no data contained to be acknowledged. This acknowledgement value enables the client to tie the response back to the original segment that it sent to the server.

3. Initiating client responds with an acknowledgement value equal to the sequence value

it received plus one. This completes the process of establishing the connection.

To  understand  the  three-way  handshake  process,  it  is  important  to  look  at  the  various values that the two hosts exchange. Within the TCP segment header, there are six 1-bit fields that contain control information used to manage the TCP processes. Those fields are:

• URG - Urgent pointer field significant.

• ACK - Acknowledgement field significant.

• PSH - Push function.

• RST - Reset the connection.

• SYN - Synchronize sequence numbers.

• FIN- No more data from sender.

These fields are referred to as flags, because the value of one of these fields is only 1 bit and, therefore, has only two values: 1 or 0. When a bit value is set to 1, it indicates what control information is contained in the segment.

Using a four-step process, flags are exchanged to terminate a TCP connection.

4.2 The TCP protocol- Communicating  with Reliability

4.2.4 TCP Three- Way Handshake

Using  the  Wireshark  outputs,  you  can  examine  the  operation  of  the   TCP  3-way handshake:

Step 1

A  TCP  client  begins  the  three-way  handshake  by  sending  a  segment  with  the  SYN

(Synchronize  Sequence  Number)  control  flag  set,  indicating  an  initial  value  in  the  sequence number  field  in  the  header.  This  initial  value  for  the  sequence  number,  known  as  the  Initial Sequence  Number  (ISN),  is  randomly  chosen  and  is  used  to  begin  tracking  the  flow  of data from the client to the server for this session. The ISN in the header of each segment is increased

by one for each byte of data sent from the client to the server as the data conversation continues.

As shown in the figure, output from a protocol analyzer shows the SYN control flag and the relative sequence number.

The  SYN  control  flag  is  set  and  the  relative  sequence  number  is  at  0.  Although  the protocol	analyzer	in	the	graphic	indicates	the	relative	values	for	the	sequence	and acknowledgement  numbers, the true  values are 32 bit binary numbers. We can determine  the actual numbers sent in the segment headers by examining the Packet Bytes pane. Here you can

see the four bytes represented in hexadecimal.

Step 2

The TCP server needs to acknowledge the receipt of the SYN segment from the client to establish the session from the client to the server. To do so, the server sends a segment back to

the  client  with  the  ACK  flag  set  indicating  that  the  Acknowledgment  number  is  significant. With  this  flag  set  in  the  segment,  the  client  recognizes  this  as  an  acknowledgement  that  the server received the SYN from the TCP client.

The  value  of  the  acknowledgment  number  field  is  equal  to  the  client  initial  sequence

number  plus  1.  This  establishes  a  session  from  the  client  to  the  server.  The  ACK  flag  will remain set for the balance of the session. Recall that the conversation between the client and the server is actually two one-way sessions: one from the client to the server, and the other from the server to the client. In this second step of the three-way handshake, the server must initiate the response from the server to the client. To start this session, the server uses the SYN flag in the same way that the client did. It sets the SYN control flag in the header to establish a session from  the  server  to  the  client.  The  SYN  flag  indicates  that  the  initial  value  of  the  sequence

number field is in the header. This value will be used to track the flow of data in this session

from the server back to the client.

As  shown  in  the  figure,  the  protocol  analyzer  output  shows  that  the  ACK  and  SYN

control flags are set and the relative sequence and acknowledgement numbers are shown.

Step 3

Finally, the TCP client responds with a segment containing an ACK that is the response

to  the  TCP  SYN  sent  by  the  server.  There  is  no  user  data  in  this  segment.  The  value  in  the acknowledgment  number  field  contains  one  more  than  the  initial  sequence  number  received from  the  server.  Once  both  sessions  are  established  between  client  and  server,  all  additional segments exchanged in this communication will have the ACK flag set.

As shown in the figure, the protocol analyzer output shows the ACK control flag set and

the relative sequence and acknowledgement numbers are shown.

Security can be added to the data network by:

• Denying the establishment of TCP sessions

• Only allowing sessions to be established for specific services

• Only allowing traffic as a part of already established sessions.

This security can be implemented for all TCP sessions or only for selected sessions.

4.2 The TCP protocol- Communicating  with Reliability

4.2.5 TCP Session Termination

To close a connection, the FIN (Finish) control flag in the segment header must be set. To

end each one-way TCP session, a two-way handshake is used, consisting of a FIN segment and

an  ACK  segment.  Therefore,  to  terminate  a  single  conversation  supported  by  TCP,  four exchanges are needed to end both sessions.

Note: In this explanation, the terms client and server are used in this description as a reference for simplicity, but the termination process can be initiated by any two hosts that complete the session:

1. When the client has no more data to send  in the stream, it sends a segment with the

FIN flag set.

2.  The  server  sends  an  ACK  to  acknowledge  the  receipt  of  the  FIN  to  terminate  the

session from client to server.

3. The server sends a FIN to the client, to terminate the server to client session.

4. The client responds with an ACK to acknowledge the FIN from the server.

When the client end of the session has no more data to transfer, it sets the FIN flag in the header  of  a  segment.  Next,  the  server  end  of  the  connection  will  send  a  normal  segment containing data with the ACK flag set using the acknowledgment number, confirming that all

the bytes of data have been received. When all segments have been acknowledged, the session

is closed.

The session in the other direction is closed using the same process. The receiver indicates that there is no more data to send by setting the FIN flag in the header of a segment sent to the source. A return acknowledgement confirms that all bytes of data have been received and that session is, in turn, closed.

As shown  in the  figure, the  FIN and  ACK control flags are  set  in the  segment  header, thereby closing a HTTP session.

It is also possible to terminate the connection by a three-way handshake. When the client

has no more data to send, it sends a  FIN to the server. If the server also has no more data to send,  it  can  reply  with  both  the  FIN  and  ACK  flags  set,  combining  two  steps  into  one.  The client replies with an ACK.

4.5 Summary and Review

The Transport layer provides for data network needs by:

•	Dividing data received from an application into segments

•	Adding a header to identify and manage each segment

•	Using the header information to reassemble the segments back into application data

•	Passing the assembled data to the correct application

UDP and TCP are common Transport layer protocols.

UDP  datagrams  and  TCP  segments  have  headers  prefixed  to  the  data  that  include  a source port number and destination port number. These port numbers enable data to be directed

to the correct application running on the destination computer.

TCP does not pass any data to the network until it knows that the destination is ready to receive it. TCP then manages the flow of the data and resends any data segments that are not acknowledged  as  being  received  at  the  destination.  TCP  uses  mechanisms  of  handshaking, timers and acknowledgements, and dynamic windowing to achieve these reliable features. This

reliability  does,  however,  impose  overhead  on  the  network  in  terms  of  much  larger  segment

headers  and  more  network  traffic  between  the  source  and  destination  managing  the  data transport.

If the  application  data  needs  to  be  delivered  across  the  network  quickly,  or  if  network

bandwidth  cannot  support  the  overhead  of  control  messages  being  exchanged  between  the source  and  the  destination  systems,  UDP  would  be  the  developer's  preferred  Transport  layer protocol.  Because  UDP  does  not  track  or  acknowledge  the  receipt  of  datagrams  at  the destination - it just passes received datagrams to the Application layer as they arrive - and does

not  resend  lost  datagrams.  However,  this  does  not  necessarily  mean  that  the  communication

itself  is  unreliable;  there  may be  mechanisms  in the  Application  layer protocols  and  services that process lost or delayed datagrams if the application has these requirements.

The choice of Transport layer protocol is made by the developer of the application to best

meet the user requirements. The developer bears in mind, though, that the other layers all play a part in data network communications and will influence its performance.